PRODUCT  DATA  SHEET

###### S-MVTF

###### MULTI-VEHICLE

###### AUTOMATIC

###### TRANSMISSION FLUID

G

PRODUCT  DESCRIPTION

In the past, automatic transmissions were relatively simple compared to today’s modern equipment, all use a basic torque converter, most had three or four speeds.  A handful of automatic transmissions fluids (ATF’s) served the entire market.  The vast majority of ATF’s applications were satisfied by DEXRON® III/ MERCON® Fluid, while had effectively become a universal fluid due to the number of vehicles specifying this type of ATF.  Today however demand for improved fuel economy, and lower emissions have driven transmission design changes.

There are multiple automatic transmissions options on almost every vehicle model regardless of size,
 the numbers, complexity and variety of automatic transmissions types in service, have grown exponentially. Because each of these transmissions types has subtly different operation requirements, OEM’s specify a different type of ATF.

As the ATF marketplace has become crowded with fluids developed to keep up with the steady stream of renovations it has created a challenge for both producer and consumers.  The many ATF’s specified also create a dilemma.  At least 30 recommended types of ATF’s are needed to service the vehicles on the road today.

The solution is here, Sentinel’s Synthetic 
S-MVTF, multi-vehicle universal transmission fluid, formulated with innovative additives to meet the rigorous requirements of multiple manufacturers of both domestic and foreign

automatic transmissions in today’s passenger cars, light trucks and SUV’s.

Sentinel® has simplified the complexity of the many transmissions fluid requirements to one product. It is truly a one of a kind fluid and eliminates the guesswork in proper ATF selection.

Sentinel’s S-MVTF is recommended for use and can be used with confidence in applications specifying the following fluid and in automatic transmissions from the following manufacturers.

- GM DEXRON® II, III, V &VI

- Ford MERCON®, MERCON® V & SP, LV

- Chrysler ATF+ through ATF +4®

- Allison C-3, C-4, Allison TES-295

- Vickers I-286S & M-2950S

*See additional list below.

###### S-MVTF

###### MULTI-VEHICLE AUTOMATIC TRANSMISSION FLUID

OEM  ATF  RECOMMENDATIONS

| OEM | MODEL | OEM RECOMMENDED FLUID |  |
| --- | --- | --- | --- |
| Acura | All Except CVT | Dexron®II Honda ATF-Z1 |  |
| Alfa Romeo |  |  |  |
| American Motors |  |  |  |
| Audi | All Except CVT | Dexron® II Audi #G-052-025-A2 |  |
| BMW | All | ESSO Type LT71141 Shell LA2634 TEXACO ETL7045E/8072B BMW 83229407765 Dexron® II/III |  |
| Cadillac |  |  |  |
| Chevrolet |  |  |  |
| Chrysler/ Dodge / Plymouth | All | Mopar® ATF +3/ +4 Shell 3403 ATF |  |
| Daewoo | All | Dexron® III |  |
| Eagle | All | Mopar® ATF +3/ +4 Dexron® II Mercon® |  |
| Ford*/ Lincoln/Mercury | All Except CVT & Type F ATF | Mercon® Mercon® V Mercon® SP Mercon® LV |  |
| General Motors | All Except CVT & ‘03+ Pontiac Vibe | Dexron ® III/V/VI Toyota Type T-IV Allison C4 (TES228), TES295, TES3289 |  |
| Geo |  |  |  |
| GMC |  |  |  |
| Honda | All Except CVT | Honda ATF-Z1 |  |
| Hyundai | All | Hyundai Genuine ATF SP-II/SP-III Mopar® ATF + 3 |  |
| Isuzu | All Except CVT | Dexron® III Honda ATF-Z1 |  |
| Jeep | All | Mopar ATF + 4 Dexron®- III/ Dexron®- III/Mercon® |  |
| Kia | All | Kia SP-II ATF Dexron®- II/III |  |
| Mazda* | All Except Type F | Dexron®- III/Mercon®/Mercon® V |  |
| Mercedes Benz | All | Dexron® III MB 0019892/10310 Shell 3403 ATF |  |
| Mitsubishi |  | Mitsubishi Diamond SP II/III ATF Dexron® II/III |  |
| Nissan* | All Except CVT | Nissan Matic- D/J/K Dexron®- II/III |  |
| Porsche | All | Porsche Part # 999 917 547 00 (A2) ESSO LT71141 | Shell 3403 ATF Dexron®- III/Mercon® |
| Saturn | All Except CVT | Saturn T-IV (22689186) Saturn Transaxle Fluid (210005966) Dexron®- III/VI |  |
| Subaru | All | Dexron®- III/Mercon® |  |
| Suzuki | All | Dexron®- III/Mercon® |  |
| Toyota* | All Except CVT | Toyota Type WS ATF Toyota Type TII/T-IV Dexron®- III/Mercon® |  |
| Volkswagen | All | Part # G 052 162 (-A1, -A2) ESSO LT71141 Dexron®- III/Mercon® |  |
| Volvo | All Except Type F | Volvo 1161540-8 Dexron®- III/Mercon® |  |

Continue

OEM ATF Recommendations

*Not for use in vehicles requiring Type F fluid, including some 984-86 model year Mazda’s, 1984-93 Nissan Matic J&K, Saab900’s, 1991-92 Jaguars, 1984-85 Toyota WS, Volvos, and (CVT’s) or in 6 speed transmissions found in BMW 7 Series.

Sentinel’s S-MVTF is not recommended for continuous variable transmissions (CV7’s) or vehicles specifying type F-ATF.

PRODUCT ADVANTAGES

| - Long oil and transmission life. - Exceptional oxidation stability. - Wide temperature range. - Resistant to chemical deterioration. - Excellent viscosity stability - Resist thermal degradation | - Provides easy starting, faster warm ups and smoother shifting. - Improved fuel economy. - Excellent seal compatibility. - Resistant to shear viscosity breakdown. - Exceptional high temperature performance. |
| --- | --- |

TYPICAL CHARACTERISTICS

Viscosity cSt    @ 100C			 	                   6.8                                                	ASTM D-     445

@   40C	                                      32	                     		ASTM D-     445

Viscosity Index  	                  179	                    	 	ASTM D-   2270

Brookfield Viscosity (cP) - 40	C		                     7000

Pour Point C (F)	                                          	          -55(-67)		 		ASTM D-       97

Noack Volatility, % weight loss                          	                   10					ASTM D-   5800

Flash Point C (F)                                            	        230(446)		                 		ASTM D-       92

Four Ball wear Test, Scar in mm: 40 kg,

75°C, 1200 rpm 1 hr           		           .40		      			ASTM D- 4172B

TO ORDER or FOR ADDITIONAL INFORMATION