SAFETY DATA SHEET

Sen-No-Freeze

Revision B

Effective Date: 11/08/2018

Regulation: 1907/2006/EC, In accordance with the provisions of Article 41, Industrial Safety & Health Act.

OSHA Hazard Communication Standard (29 CFR 1910.1200).

1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - PRODUCT IDENTIFIER	: Material Name: Sen-No-Freeze		Product Code	: 02120

  - RELEVANT IDENTIFIED USES OF THE SUBSTANCE OR MIXTURE AND USES ADVISED AGAINST

Product Use		: Anti-Freeze/Engine Coolant

Uses Advised Against	: This product must not be used in applications other than those 					                recommended in Section 1, without first seeking the advice of the supplier.

  - DETAILS OF THE SUPPLIER OF THE SAFETY DATA SHEET

Manufacturer/Supplier	: Sentinel Lubricants Inc.

15755 NW 15th Ave

Miami, FL 33169

Telephone			: Marketing Technician Department

1(800) 842-6400, (305) 625-6400         		 Fax      : (305) 625-6565

Email Contact for Safety Data Sheet	: info@sentinelsynthetic.com

1.4 EMERGENCY TELEPHONE NUMBER	: INFOTRAC – 1.800.535.5053     Contract #107464

International – 352.323.3500

2. HAZARDS IDENTIFICATION

2.1 HAZARD CLASSIFICATION 	: Not Classified

2.2 SIGNAL WORD			: None

2.3 HAZARD PICTOGRAMS 		: None

2.4 HAZARD STATEMENTS	: None

2.5 PRECAUTIONARY STATEMENTS	: None

3. COMPOSITION/INFORMATION ON INGREDIENTS

3.1 INGREDIENTS

| Chemical Identity |  | CAS |  | % wt Range |
| --- | --- | --- | --- | --- |
| Propylene Glycol |  | 57-55-5 |  | 80-95 |
| Water |  | 7732-18-5 |  | 5-20 |

As per paragraph (i) of 29 CFR 1910.1200, formulation is considered a trade secret and specific chemical identity and exact percentage of composition may have been withheld.

4. FIRST AID MEASURES

4.1 DESCRIPTION OF FIRST AID MEASURES

General Information	: If exposed or concerned, seek medical advice or attention.

Inhalation	: Remove to fresh air. If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing. Contact a poison center or a doctor if you feel unwell.

Skin Contact	: Not expected to present a significant skin hazard under normal use.

Eye Contact	: Immediately flush with clear water for at least 15 minutes, including under the eyelids. Consult a doctor.

Ingestion	: Immediately have the victim drink plenty of water. Keep airway free. Contact a physician. Never give anything by mouth if victim is rapidly losing consciousness, unconscious or convulsing.

Self-protection

of the first aider	: Wear adequate personal protective equipment based on the nature and severity of the emergency.

4.2 MOST IMPORTANT SYMPTOMS AND EFFECTS, BOTH ACUTE AND DELAYED

Symptoms/Injuries	: Not expected to present a significant hazard under anticipated conditions of normal use.

Symptoms after skin contact	: Prolonged contact may cause slight irritation.

Symptoms after eye contact	: May cause slight irritation.

Symptoms after ingestion	: Excessive ingestion may cause central nervous system effects.

5. FIRE FIGHTING MEASURES

5.1 EXTINGUISHING MEDIA	: Use alcohol resistant foam, carbon dioxide, dry chemical or water spray when fighting fires. Do not direct a water stream directly into the hot burning liquid.

5.2 FIRE OR EXPLOSION HAZARDS	: None, reactivity is stable.

5.3 HAZARDOUS COMBUSTION PRODUCTS	: Oxides of carbon (CO, CO2).

5.4 ADVICE FOR FIREFIGHTERS	: Do not enter fire area without proper protection including self-contained breathing apparatus and full protective equipment. Fight fire from a safe distance and a protected location due to the potential of hazardous vapors and decomposition products. Use appropriate methods for the surrounding fire.

Protective Equipment	: As with any fire, wear SCBA pressure-demand, MSHA/NIOSH approved and full protective gear.

6. ACCIDENTAL RELEASE MEASURES

6.1 PERSONAL PRECAUTIONS, PROTECTIVE EQUIPMENT AND EMERGENCY PROCEDURES

For non-emergency personnel	: No action should be taken by non-emergency personnel without suitable training. Evacuate surrounding areas. Keep unnecessary and unprotected personnel from entering. Do not touch or walk through spill.

For emergency responders	: Use personal protection as recommended in Section 8. Observe precautions provided for non-emergency personnel.

6.2 ENVIRONMENTAL PRECAUTIONS	: Use appropriate containment to avoid environmental contamination. Prevent from spreading or entering drains, ditches or rivers by using sand, earth or other appropriate barriers.

6.3 METHODS AND MATERIAL FOR CONTAINMENT AND CLEANING UP

Cleanup Procedures	: Contain spill. Soak up material with inert absorbent and place in safety containers for proper disposal.

6.4 REFERENCE TO OTHER SECTIONS	: For guidance on selection of personal protective equipment see Section 8. For guidance on disposal of spilled material see Section 13.

7. HANDLING AND STORAGE

7.1 PRECAUTIONS FOR SAFE HANDLING

General Handling Precautions	: KEEP OUT OF REACH OF CHILDREN. Avoid prolonged or repeated eye, skin or clothing contact. Provide good ventilation. Avoid breathing of vapors or mists. Wash hands after use. Do not take internally. Industrial use only.

Hygiene Recommendations	: Do not eat, drink or smoke when using this product. Wash hands thoroughly after use. Remove contaminated clothing and protective equipment before entering eating or smoking areas.

7.2 CONDITIONS FOR SAFE STORAGE

Storage Requirements	: Storage of individual cans should be done in a cool dry place. Ensure can is in a secure place to prevent knocking over and accidental rupture. Keep container tightly closed in a cool, well-ventilated place away from ignition sources. Keep away from food and drink.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1 CONTROL PARAMETERS	: No additional information available

8.2 INDIVIDUAL PROTECTION MEASURES

Respiratory Protection    	: If ventilation is not sufficient, an approved respirator with organic vapor cartridge may be permissible under certain circumstances where airborne concentrations are expected to exceed occupational exposure limits. If respirators are needed, compliance with OSHA standard 29 CFR 1910.134 is necessary.

Skin Protection            	: None required for normal conditions of use.

Eye/Face Protection	: Safety glasses with side shields are recommended as a minimum for any type of industrial chemical handling. Where contact with this material could occur, chemical splash proof goggles are recommended.

Other Protective Equip.	: Safety showers and eye-wash stations should be available in the workplace near where the material will be used.

9. PHYSICAL AND CHEMICAL PROPERTIES

9.1 PHYSICAL PROPERTIES

| Boiling Point : 154°C (310°F) | Freezing Point : NDA |
| --- | --- |
| Flash Point : 104°C (219°F) | Percent Volatile : 100 @ 1 atm 25°C (77°F) |
| Explosive Limits : 2.6% lower – upper 12.5% (V) | Autoignition Temp, Liq. : NDA |
| Flammability : Not | Relative Density (H2O=1) : 1.038 @ (15.6/15.6°C) |
| Molecular Weight : Not Determined | Weight : 8.7 lbs/gal |
| Vapor Pressure : < 0.1 mm Hg @ 20°C | pH : Essentially neutral 7 |
| Vapor Density : 2.62 (Air=1) | Evaporation Rate :< 0.01 (n-Butyl Acetate =1) |
| Form : Liquid | Partition Coefficient : N/A |
| Viscosity : NDA | Refractive Index : N/A |
| Odor Threshold : NDA | Heat of Combustion (∆Hc) : N/A |
| Odor : Odorless | Water Solubility : Soluble |
| Appearance/Color : Yellow Clear | Decomposition Temp : N/A |

10. STABILITY AND REACTIVITY

10.1 REACTIVITY	: No dangerous reaction known under conditions of normal use.

10.2 CHEMICAL STABILITY	: This product is stable under normal conditions. Hazardous polymerization will not occur.

10.3 HAZARDOUS REACTIONS	: Under normal conditions of storage and use, hazardous reactions are not expected to occur. Stable.

10.4 CONDITIONS TO AVOID	: Heat, open flame, sparks.

10.5 INCOMPATIBLE MATERIALS	: Strong oxidizing agents, strong acids, strong alkalis.

10.6 DECOMPOSITION PRODUCTS   	: Carbon monoxide, carbon dioxide.

11. TOXICOLOGICAL INFORMATION

11.1 ACUTE EXPOSURE

Eye Irritation		: Moderate to strong eye irritant, based on data from similar materials.

Skin Irritation	: Severe skin irritant, based on data from similar materials. Prolonged or repeated skin contact as from clothing wet with material may cause dermatitis. Symptoms may include redness, edema, drying and cracking of the skin.

Respiratory Irritation	: Severe nose, throat and lung irritant, based on data from similar materials. Exposure to a high concentration of vapor or mist may cause severe irritation to the nose and upper respiratory tract. Acute overexposure can cause serious nervous system depression.

Swallowing	: Harmful or fatal if swallowed. Swallowing can cause abdominal irritation, nausea, vomiting and diarrhea. The symptoms of chemical pneumonitis may not show up for a few days.

Inhalation	: May cause drowsiness or dizziness with narcotic effects.

11.2 ACUTE TOXICITY

| Material | Oral LD₅₀ |  | Dermal LD₅₀ |  | ATE US |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Value | Species | Value | Species | Value |  |
| Propylene Glycol | 20,000 mg/kg | rat | 22,500 mg/kg | rat | Oral | 20,000 mg/kg bodyweight |
|  |  |  | 20,800 mg/kg | rabbit | Dermal | 20,800 mg/kg bodyweight |

Dermal Sensitizer	: NDA to indicate that this product may be a skin sensitizer.

Inhalation Sensitizer	: NDA to indicate that this product may be a respiratory sensitizer.

11.3 HEALTH HAZARD CLASSIFICATION

Skin Corrosion / Irritation		: Classification criteria not met.

Eye Damage / Irritation    	: Classification criteria not met.

Respiratory Irritation       	: Classification criteria not met.

Respiratory / Skin Sensitization	: Classification criteria not met.

Germ Cell Mutagenicity		: Classification criteria not met.

Reproductive Toxicity		: Classification criteria not met.

STOT – Single Exposure		: Classification criteria not met.

STOT – Repeated Exposure		: Classification criteria not met.

Aspiration Hazard			: Classification criteria not met.

Carcinogen Data

| Calif Prop-65 | OSHA | NIOSH | ACGIH | NTP | IARC |
| --- | --- | --- | --- | --- | --- |
| No | No | No | No | No | No |

12. ECOLOGICAL INFORMATION

12.1 AQUATIC TOXICITY – Propylene Glycol

| FISH |  |  | INVERTEBRATES |  |  | AQUATIC PLANTS |  |  | MICROORGANISMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Value | Period | Type | Value | Period | Type | Value | Period | Type | Value | Period |
| LC50 | 51,400 mg/L | 96hr | EC50 | 34,400 mg/L | 48hr | TL | 15,000 mg/L | 336hr |  | ND |  |

12.2 PERSISTENCE AND DEGRADABILITY – Propylene Glycol

BIOCHEMICAL OXYGEN DEMAND (BOD)	: 0.96 – 1.08g O₂/g substance.

CHEMICAL OXYGEN DEMAND (COD)		: 1.63g O₂/g substance.

ThOD						: 1.69g O₂/g substance.

BOD (% of ThOD)				: 0.57% ThOD.

12.3 ECOLOGICAL DATA – Propylene Glycol

DEGRADABILITY		: This product is readily biodegradable in water.

MOBILITY IN SOIL	: This material is a biodegradable in soil. Surface tension is 0.036 N/m (25°C)

ACCUMULATION	: This product does not accumulate or bio-magnify in the environment. Log Pow is -1.41 to -0.30

12.4 OTHER ADVERSE EFFECTS	: Workplace practices must be aimed at eliminating environmental contamination. No known effect on the ozone layer. No known ecological damage caused by this product to effect global warming.

13. DISPOSAL CONSIDERATIONS

13.1 WASTE TREATMENT METHODS

Waste Disposal	: Characteristics and waste stream classification can change with product use and location. It is the responsibility of the user to determine the proper storage, transportation, treatment and/or disposal methodologies for materials and residues at the time of disposition. All waste must be disposed of in compliance with the respective national, federal, state and/or local regulations.

14. TRANSPORTATION INFORMATION

14.1 DOT			: Not regulated

14.2 IATA (Air)		: Not regulated

14.3 IMO (Ocean)		: Not regulated

15. REGULATORY INFORMATION

15.1 FEDERAL REGULATIONS

TSCA 8 (b) Inventory Status	: All components are listed or are exempt from listing on the Toxic Substances Control Act Inventory.

CERCLA/SARA Sec 313	: Not classified.

SARA 311/312	: Not classified.

16. OTHER INFORMATION

NFPA Hazard ID	: Health – 0          Flammability – 1       Reactivity – 0

HMIS Hazard ID	: Health – 0          Flammability – 1       Physical Hazard – 0

SDS Distribution	: The information in this document should be made available to all who may handle the product.

SDS Revision Number	: B

SDS Effective Date	: 11/01/2018

SDS Regulation	: Regulation 1907/2006/EC as amended by Regulation (EU) 453/2000.

In accordance with the provisions of Article 41, Industrial Safety & Health Act and OSHA Hazard Communication Standard (29 CFR 1910.1200).

Disclaimer	: The information is based on our current knowledge and is intended to describe the product for the purpose of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.