SAFETY DATA SHEET

SPO “S”

Revision B

Effective Date: 11/01/2018

Regulation: According to Federal Register/ Vol. 77, No.58/ Monday, March 26, 2012/Rules and Regulations

1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - PRODUCT IDENTIFIER

Material Name		: SPO “S”

Product Code		: 01017

  - RELEVANT IDENTIFIED USES OF THE SUBSTANCE OR MIXTURE AND USES ADVISED AGAINST

Product Use		: Synthetic Penetrating Oil

Uses Advised Against	: This product must not be used in applications other than those 					                                   recommended in Section 1, without first seeking the advice of the supplier.

  - DETAILS OF THE SUPPLIER OF THE SAFETY DATA SHEET

Manufacturer/Supplier	: Sentinel Lubricants Inc.

15755 NW 15th Ave

Miami, FL 33169

Telephone			: Marketing Technician Department

1(800) 842-6400, (305) 625-6400

Fax                                               	: (305) 625-6565

Email Contact for Safety Data Sheet: info@sentinelsynthetic.com

1.4 EMERGENCY TELEPHONE NUMBER	: INFOTRAC – 1.800.535.5053     Contract #107464

International – 352.323.3500

2. HAZARDS IDENTIFICATION

2.1 CLASSIFICATION OF THE SUBSTANCE OR MIXTURE

| Flam. Aerosol 1 | H222 | Physical Hazards | Extremely Flammable aerosol Cat. 1 |
| --- | --- | --- | --- |
| Press. Gas (Comp.) | H280 | Physical Hazards | Gases under pressure, may explode if heated |
| Skin Irrit. 2 | H315 | Health Hazards | Causes skin irritation |
| Asp. Tox 1 | H304 | Health Hazards | May be fatal if swallowed and enters airways |
| Aquatic Acute 1 | H400 | Environmental Hazards | Hazardous to the aquatic environment – Acute Hazard Cat. 1 |
| Aquatic Chronic 2 | H411 | Environmental Hazards | Hazardous to the aquatic environment – Chronic Hazard Cat. 2 |

2.2 LABEL ELEMENTS

Hazard Pictograms

| GHS02 | GHS04 | GHS07 | GHS08 | GHS09 |
| --- | --- | --- | --- | --- |

Signal Word	 : Danger

| Hazard Statements | H222 | Extremely flammable aerosol |
| --- | --- | --- |
|  | H280 | Contains gas under pressure; may explode if heated |
|  | H304 | May be fatal if swallowed and enters airways |
|  | H315 | Causes skin irritation |
|  | H400 | Very toxic to aquatic life |
|  | H411 | Toxic to aquatic life with long lasting effects |
| Precautionary Statements | P210 | Keep away from heat/sparks/open flames/hot surfaces. – No smoking |
|  | P211 | Do not spray on an open flame or other ignition source |
|  | P251 | Pressurized container: Do not pierce or burn, even after use |
|  | P264 | Wash hands thoroughly after handling |
|  | P273 | Avoid release to the environment |
|  | P280 | Wear protective gloves and eye protection |
|  | P301+P310 | If swallowed: immediately call POISON CENTER |
|  | P302+P352 | If on skin: wash with plenty of water |
|  | P331 | Do NOT induce vomiting |
|  | P332+P313 | If skin irritation occurs: Get medical advice/attention |
|  | P362+P364 | Take off contaminated clothing and wash it before reuse |
|  | P391 | Collect spillage |
|  | P403 | Store in a well-ventilated place |
|  | P410+P412 | Protect from sunlight. Do not expose to temperatures exceeding 50°C/122°F |
|  | P501 | Dispose of contents/container to applicable regulations |

2.3 OTHER HAZARDS WHICH DO NOT RESULT IN CLASSIFICATION

Unclassified Hazards			: None identified.

2.4 UNKNOWN ACUTE TOXICITY

20.76% of the mixture consists of ingredients of unknown acute toxicity (Oral)

40.76% of the mixture consists of ingredients of unknown acute toxicity (Dermal)

22.76% of the mixture consists of ingredients of unknown acute toxicity (Inhalation (Vapours)

3. COMPOSITION/INFORMATION ON INGREDIENTS

3.1 SUBSTANCE / MIXTURE

Substance / Mixture		: Mixture

3.2 COMPOSITION

| Substance Name | CAS | % wt Range | Classification |
| --- | --- | --- | --- |
| Stoddard Solvent | 8052-41-3 | 10-30 | Flam. Liq. 3, H226 Asp. Tox. 1, H304 |
| Propane | 74-98-6 | 5-10 | Flam. Gas 1, H220 Press. Gas (Diss.., H280 |
| N-Heptane | 142-82-5 | 10-30 | Flam. Gas 1, H225 Skin Irrit. 2, H315 STOT SE 3, H336 Asp. Tox. 1, H304 Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |
| N-Butane | 106-97-8 | 5-10 | Flam. Gas 1, H220 Press. Gas (Diss), H280 |
| Isobutane | 75-28-5 | 1-5 | Flam. Gas 1, H220 Press. Gas (Diss.., H280 |

4. FIRST AID MEASURES

4.1 DESCRIPTION OF FIRST AID MEASURES

General Information	: Call a physician immediately.

Inhalation	: Remove person to fresh air and keep comfortable for breathing.

Skin Contact	: Wash skin with plenty of water.  Take off contaminated clothing.  If skin irritation occurs:  Get medical advice/attention.

Eye Contact	: Rinse eye with water as a precaution.

Ingestion	: Do not induce vomiting. Call a physician immediately.

First-Aid 	: Wear adequate personal protective equipment based on the nature and severity of the

Responder Protection          emergency.

4.2 MOST IMPORTANT SYMPTOMS AND EFFECTS, BOTH ACUTE AND DELAYED

Symptoms of Exposure	: Eye irritation, Nose Irritation, Lassitude (Weakness), Dermatitis, Confusion, Skin Irritation, Headache, Dizziness, Nausea, Narcosis, Vomiting, Cough, Mucous Membrane, Diarrhea.

Delayed Effects	: No known delayed effects.

Immediate Effects	: No known immediate effects.

Chronic Effects	: No known chronic effects.

Target Organs	        : Central Nervous System, Eyes, Liver, Nasal Cavity, Reproductive System, Respiratory

System, Skin, Kidneys.

4.3 INDICATION OF ANY IMMEDIATE MEDICAL ATTENTION AND SPECIAL TREATMENT NEEDED

Note to Physician			: Treat symptomatically

Specific Treatments / Antidotes	: No information available.

Immediate Medical Attention	: May aggravate personnel with pre-existing disorders associated with any of the target organs.

5. FIRE FIGHTING MEASURES

5.1 EXTINGUISHING MEDIA

Extinguishing Media		: Carbon dioxide, dry chemical, Product is non-flammable.  Use extinguishing                                                                                                                                       media suitable for surrounding materials, universal aqueous film forming foam.

Unsuitable Media		: Water jet

5.2 SPECIAL HAZARDS ARISING FROM THE SUBSTANCE OR MIXTURE

Decomposition Products	                : Decomposition products may include: oxides of carbon, smoke, vapors.

Hazards from The Product	: Extremely flammable.  Contents under pressure.  In a fire or if heated, a   pressure increase will occur which may result in container bursting.  Vapors heavier than air may spread along the ground and travel to an ignition source.

5.3 SPECIAL PROTECTIVE ACTIONS FOR FIRE FIGHTERS

Protective Action	: Use water spray to cool fire exposed containers as contents may rupture violently from heat developed pressure.

Protective Equipment 	: Firemen should wear self-contained breathing apparatus with full face-piece                            operated in positive pressure mode.

6. ACCIDENTAL RELEASE MEASURES

6.1 PERSONAL PRECAUTIONS, PROTECTIVE EQUIPMENT AND EMERGENCY PROCEDURES

For non-emergency personnel	: No action should be taken involving any personnel without suitable training.  Evacuate surrounding areas.  Keep unnecessary and unprotected personnel from entering.  Do not touch or walk through spill.  Remove ignition sources and provide adequate ventilation only if it is safe to do so.

For emergency responders	: Use personal protection as recommended in Section 8. Observe precautions provided for non-emergency personnel.

6.2 ENVIRONMENTAL PRECAUTIONS

Keep out of drains, sewers, ditches and waterways. Minimize use of water to prevent environmental contamination.

6.3 METHODS AND MATERIAL FOR CONTAINMENT AND CLEANING UP

Containment Procedures	: Product is an aerosol, therefore spills and leaks are unlikely. In case of rupture, released content may be contained with oil/solvent absorbent pads, socks and/or absorbents.

Cleanup Procedures	: Spills from aerosol cans are unlikely and are generally of small volume. Large spills are therefore not normally considered a problem. In case of actual rupture, avoid breathing vapors and ventilate area well. Remove sources of ignition and use non-sparking equipment. Soak up material with inert absorbent and place in safety containers for proper disposal.

Other Information	: Aerosol products represent a limited hazard and will not spill or leak unless ruptured. In case of rupture contents are generally evacuated from the can rapidly. Area should be ventilated immediately and continuous ventilation provided until all fumes and vapors have been removed. Aerosol cans should never be incinerated or burned.

Prohibited Materials	: Combustible absorbent material such as sawdust, use of equipment that may cause sparking.

7. HANDLING AND STORAGE

7.1 PRECAUTIONS FOR SAFE HANDLING

General Handling Precautions	: KEEP OUT OF REACH OF CHILDREN. Avoid prolonged or repeated skin contact. Avoid breathing of vapors. Do not incinerate containers. Always replace overcap when not in use. Avoid use around open flames or sources of ignition. Exposure to heat or prolonged exposure to sun may cause can to burst. Use only with adequate ventilation, opening doors or windows to achieve cross-ventilation.

Hygiene Recommendations	: Do not eat, drink or smoke when using this product.  Remove contaminated clothing and protective equipment before entering eating or smoking areas. Wash hands thoroughly after use.

7.2 CONDITIONS FOR SAFE STORAGE INCLUDING ANY INCOMPATIBILITIES

Storage Requirements	: Storage of individual cans should be done in an area below 55°C (120°F) and away from heat sources. Ensure can is in a secure place to prevent knocking over and accidental rupture. For storage of pallet quantities, compliance with NFPA 30B (Manufacture and Storage of Aerosol Products) is recommended.

Incompatibilities		       : Segregate storage away from materials indicated in Section 10.

NFPA 30B Classification                      : This product is classified as a level 3 aerosol per NFPA 30B.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1 CONTROL PARAMETERS

| N-Heptane (142-82-5) |  |  |
| --- | --- | --- |
| ACGIH | ACGIH TWA (mg/m³) | 400 ppm |
| OSHA | OSHA PEL (TWA) (mg/m³) | 2000 mg/m³ |
| OSHA | OSHA PEL (TWA) (ppm) | 500 ppm |
| NIOSH | US IDLH (ppm) | 750 ppm |
| NIOSH | NIOSH REL (TWA) (mg/m³) | 350 mg/m³ |
| NIOSH | NIOSH REL (TWA) (ppm) | 85 ppm |
| NIOSH | NIOSH REL (ceiling) (mg/m³) | 1800 mg/m³ |
| NIOSH | NIOSH REL (ceiling) (ppm) | 440 ppm |
| California | California PEL (TWA) (mg/m³) | 1600 mg/m³ |
| California | California PEL (TWA) (ppm) | 400 ppm |
| California | California PEL (STEL) (mg/m³) | 2000 mg/m³ |
| California | California PEL (STEL) (ppm) | 500 ppm |
| N-Butane (106-97-8) |  |  |
| ACGIH | ACGIH TWA (mg/m³) | 1000 ppm |
| NIOSH | NIOSH REL (TWA) (mg/m³) | 1900 ppm |
| NIOSH | NIOSH REL (TWA) (ppm) | 800 ppm |
| California | California PEL (TWA) (mg/m³) | 1900 mg/m) |
| California | California PEL (TWA) (ppm) | 800 ppm |
| Propane (74-98-6) |  |  |
| OSHA | OSHA PEL (TWA) (ppm) | 1000 ppm |
| NIOSH | US IDLH (ppm) | 2100 ppm |
| NIOSH | NIOSH REL (TWA) (mg/m³) | 1800 mg/m³ |
| NIOSH | NIOSH REL (TWA) (ppm) | 1000 ppm |
| California | California PEL (TWA) (mg/m³) | 1800 mg/m³ |
| California | California PEL (TWA) (ppm) | 1000 ppm |
| Isobutane (75-28-5) |  |  |
| ACGIH | ACGIH TWA (mg/m³) | 1000 ppm |
| NIOSH | NIOSH REL (mg/m³) | 1900 mg/m³ |
| NIOSH | NIOSH REL (TWA) (ppm) | 800 ppm |
| Stoddard Solvent(8052-41-3) |  |  |
| ACGIH | ACGIH TWA (ppm) | 100 ppm |
| OSHA | OSHA PEL (TWA) (mg/m³) | 2900 mg/m³ |
| OSHA | OSHA PEL (TWA) (ppm) | 500 ppm |
| California | California PEL (TWA) (mg/m³) | 525 mg/m³ |
| California | California PEL (TWA) (ppm) | 100 ppm |

8.2 EXPOSURE CONTROLS

Engineering Measures	: Use only with adequate ventilation. General ventilation (typically 10 air changes per hour) should be used. Ventilation rates should be matched to conditions. Local exhaust ventilation or an enclosed handling system may be necessary to control air contamination below that of the lowest OEL from the table above.

Personal Protective

Equipment

Eye/Face Protection	: Safety glasses with side shields are recommended as a minimum for any type of industrial chemical handling. Where eye contact with this material could occur, chemical splash proof goggles are recommended.

Hand Protection		: Chemical-resistant gloves, tested according to ASTM F903-17.

Remarks	: Choose gloves to protect hands against chemicals depending on the concentration and   quantity of the hazardous substance and specific to the place of work.

Skin and Body Protection          : For brief contact, no precautions other than clean body covering clothing should be needed.  When prolonged or repeated contact could occur, use protective clothing impervious to the ingredients listed in Section 2.

Respiratory Protection              : An approved respirator with organic vapor cartridge may be permissible under certain circumstances where airborne concentrations are expected to exceed occupational exposure limits.

Compliance	: If needed, compliance with OSHA standard 29 CFR 1910.134 is necessary.

Other Protective Equip.	: Safety showers and eye-wash stations should be available in the workplace near where the material will be used.

Environmental Exposure

Controls	: Avoid release to the environment

9. PHYSICAL AND CHEMICAL PROPERTIES

9.1 PHYSICAL PROPERTIES

| Boiling Point : >93.00°C | Melting/Freezing Point : >-100.00°C |
| --- | --- |
| Flash Point : >-9.00°C | Flash Point, Propellant : -104.40°c |
| Explosive Limits : LEL: 0.60 UEL: 10.00 vol % | Autoignition Temp, Liq. : >236.00°C |
| Flammability : Extremely Flammable Aerosol | Density : 0.727 g/cm³ |
| Molecular Weight : N/A | Weight : 6.067 lbs./gal |
| Vapor Pressure : N/A | pH : N/A |
| Vapor Density : N/A | Evaporation Rate (nBAc=1) : N/A |
| Viscosity : N/A | Pertition Coefficient (Log Pow) : N/A |
| Odor Threshold : N/A | Refractive Index : N/A |
| Physical State :Pressurized Product | Heat of Combustion :19193.12 BTU/lb |
| Odor : Petroleum-like | Water Solubility : N/A |
| Appearance/Color : Straw colored liquid | Decomposition Temperature : N/A |

9.2 ENVIRONMENTAL PROPERTIES

| Percent Volatile : 71.00 % Wt | VOC Regulatory : 516.06 g/L (4.31 lbs/gal) |
| --- | --- |
| Percent VOC : 71.00 % Wt | VOC Actual : 516.17 g/L (4.31 lbs/gal) |
| Percent HAP : 0.03 % Wt | HAP Content : 0.22 g/L (0.00 lbs./gal) |
| Ozone Depletion Potential : 0.00 ODP | Maximum Incremental Reactivity :0.8800 g 03/g |

10. STABILITY AND REACTIVITY

10.1 REACTIVITY	: No specific test data related to reactivity is available for this product or its ingredients.

10.2 CHEMICAL STABILITY		: This product is stable.

10.3 HAZARDOUS REACTIONS	: Under normal conditions of storage and use, hazardous reactions are not expected to occur.

10.4 CONDITIONS TO AVOID		: Under normal conditions of storage and use, hazardous reactions are not  expected to occur.

10.5 INCOMPATIBLE MATERIALS	: Strong Oxidizing Agents, Strong Reducing Agents, Alkali Metals, Strong Acids, Aluminum, Halogen Compounds, Bases, Calcium Hypochlorite, Aluminum Chloride, Acids, Sulfuric Acid, Perchloric Acid, Chlorosulfuric Acid, Potassium Chlorate, Heavy Metals, and their Salts, Phenols, Perfomic Acid.

10.6 DECOMPOSITION PRODUCTS          : Oxides of Carbon, Aldehydes, Peroxybenzoic Acid, Benzoic Acid.

11. TOXICOLOGICAL INFORMATION

11.1 INFORMATION ON TOXICOLOGICAL EFFECTS

| Isobutane CAS: 175-28-5 |  |
| --- | --- |
| LC50 Inhalation (Rat) | 368000 ppm/4h (Cheminfo) |
| N-Heptane CAS: 142-82-5 |  |
| LD50 Oral (Rat) | 15000 mg/kg (RTECS) |
| LD50 Dermal (Rabbit) | >3160 mg/kg (Lit.) |
| LC50 Inhalation (Rat) | 25132 mg/l/4h 103 gm/m³ (RTECS) |
| N-Butane CAS: 106-97-8 |  |
| LC50 Inhalation (Rat) | 276000 ppm/4h (Cheminfo) |
| LC50 Inhalation (Rat) | 658 mg/l/4h (Cheminfo) |
| Propane Cas: 74-98-6 |  |
| LC50 Inhalation (Rat) | 658 mg/l/4h (Lit.) |
| Stoddard Solvent (8052-41-3 |  |
| LD50 Oral (Rat) | >5000 mg/k (RTECS) |

11.1.2 HEALTH HAZARD CLASSIFICATION

Routes of Exposure			: Eye contact, Ingestion, Skin Contact, Inhalation

Delayed & Immediate Effects		: See section 4.2 (also includes Chronic Effects from short and long term exposure)

Skin Corrosion / Irritation		: Causes skin irritation

Eye Damage / Irritation    	: Not Classified

Respiratory / Skin Sensitization	: Not Classified

Germ Cell Mutagenicity		: Not Classified

Reproductive Toxicity		: Not Classified

STOT – Single Exposure		: Not Classified

STOT – Repeated Exposure		: Not Classified

Aspiration Hazard			: May be fatal if swallowed and enters airways

Carcinogen Data			: None of the ingredients in the product are listed with OSHA, IARC, NTP or ACGIH                                                as being suspected or known carcinogen in a concentration greater than 0.1% by weight.

12. ECOLOGICAL INFORMATION

12.1 ECOTOXICITY AND ECOLOGICAL PROPERTIES

| N-Heptane CAS: 142-82-5 |  |
| --- | --- |
| LC50 fish 1 | 375 mg/l 96h, Mozambique Tilapia (Lit.) |
| EC50 Daphnia | 0.2mg/l 48h, Leach (Lit.) |
| Persistence and degradability | Readily biodegradable in water, Biodegradability in soil: N/A, absorbs into the soil. |
| Biochemical oxygen demand (BOD) | 1.92 g O2/g substance |
| Chemical oxygen demand (COD) | 0.06 g O2/g substance |
| ThOD | 3.52 g O2/g substance |
| Log Pow | 4.66 (Experimental value) |
| Bioacculative potential | Potential for bioaccumulation (4 ≥ Log Kow ≤5). |
| Stoddard Solvent CAS: 8052-41-3 |  |
| LC50 Fish 1 | Rainbow Trout – 96 hr |
| Log Pow | 3.16-7.06 |
| Log Koc | Log Koc, 2.85-6.74 |
| n-Butane CAS: 106-97-8 |  |
| Bioaccumulative potential | Low potential for bioaccumulation (Log Kow <4). |
| Persistence and degradability | Readily biodegradable in water. |
| Bioconcentration Factor | 33.52 |
| Log Pow | 2.89 |
| Bioacculative Potential | Low Potential for bioaccumulation (Log Kow <4) |
| Log Koc | 1.641 |
| Propane CAS: 74-98-6 |  |
| Persistence and degradability | Readily biodegradable in water. Photodegradation in the air |
| BCF Fish 1 | 9 – 25 (BCF) |
| Log Pow | 2.28 (Calculated) |
| Bioaccumulative potential | Low potential for bioaccumulation (Log Kow <4). |
| Isobutane CAS: 75-28-5 |  |
| Persistence and degradability | Readily biodegradable in water. Biodegradable in the soil. N/A (gas). |
| BCF Fish | 26.62 |
| Log Pow | 2.76 |
| Bioaccumulative potential | Low potential for bioaccumulation (BCF <500). |
| Log Koc | 1.545 |

13. DISPOSAL CONSIDERATIONS

13.1 WASTE TREATMENT METHODS

Waste Disposal	: Characteristics and waste stream classification can change with product use and location. It is the responsibility of the user to determine the proper storage, transportation, treatment and/or disposal methodologies for solvent materials and residues at the time of disposition. All waste must be disposed of in compliance with the respective national, federal, state and/or local regulations.

Container Disposal	: In the USA, an aerosol container that does not contain a significant amount of liquid would meet the definition of scrap metal [40 CFR 261.1(c)(6)] and would be exempt from RCRA regulation under 40 CFR 261.6 (a)(3)(iv) if it is to be recycled. If containers are to be disposed of (not recycled) it must be managed under all applicable RCRA and state regulations.

Landfill Precautions		: Not available

Incineration Precautions	: ** Do not incinerate; contents under pressure **.

14. TRANSPORTATION INFORMATION

| Transportation Information | DOT (USA) | IATA (AIR) | IMDG (OCEAN) |
| --- | --- | --- | --- |
| 14.1 UN Number | UN-1950 | UN-1950 | UN-1950 |
| 14.2 UN Proper Shipping Name | Aerosols, Limited Quantity | Aerosols, Flammable, Ltd. Qty. | Aerosols, Limited Quantity |
| 14.3 Transport Hazard Class(es) Labels Limited Qty. EmS Code | 2.1 None Yes N/A | 2.1 2.1 – Flammable gas Yes N/A | 2.1 None Yes F-D, S-U |
| 14.4 Packaging Group | --- | --- | --- |
| 14.5 Environmental Hazards | No | No | No |
| 14.6 Special Precautions | None | None | None |
| 14.7 Transport in Bulk | N/A | N/A | N/A |

15. REGULATORY INFORMATION

15.1 FEDERAL REGULATIONS

| SARA Section 313 | Chemical (s) subject to the reporting requirements of Section 313 or Title III of the Superfund Amendments and Reauthorization Act (SARA) of 1986 and 40CFR Part 372. Benzene; CAS: 71-43-2; <1%. Naphthalene; CAS: 91-20-3; <1%. Cumene; CAS: 98-82-8; <1%. Ethyl Benzene; CAS: 100-41-4; <1%. Toluene; CAS: 108-88-3; <1%. |
| --- | --- |
| TSCA Section 12(b) | This product or mixture is not known to contain a chemical or chemicals subject to the export notification requirements of section 12(b) of the Toxic Substances Control Act (TSCA) and 40 CFR Part 707, subpart D |
| CERCLA Reportable Quantity | Chemical(s) subject to reporting requirements of Section 102 of the Comprehensive Environmental Response, Compensation, and Liability Act (CERCLA) if released to the environment at or above the reportable quantity. Benzene; CAS: 71-43-2; 10 lb. Naphthalene; CAS: 91-20-3; 100 lb. Cumene; CAS: 98-82-8; 5000 lb. Ethyl Benzene; CAS: 100-41-4; 1000 lb. Toluene; CAS: 108-88-3; 1000 lb. |

15.2 STATE REGULATIONS

| California Proposition 65 | This product does not contain any substance known to the State of California to cause cancer, developmental and/or reproductive harm. Benzene; Cancer; Yes; 0.0002%. Naphthalene; Cancer; Yes; 0.0%. Cumene; Cancer; Yes; 0.0002%. Ethyl Benzene; Cancer; Yes; 0.0002%. Toluene; Developmental Toxicity; Yes; 0.0033%. Ethyl Benzene; No significance risk level (NSRL); 54. Toluene; No Significance risk level (NSRL); 7000. |
| --- | --- |
| State Right-to-Know Lists | The following chemical (s) appear on one or more state RTK (Right to Know) lists as indicated n-Butane (106-97-8); USA – NJ – Right to Know Hazardous Substance List. Propane (74-98-6); USA – NJ – Right to Know Hazardous Substance List. Isobutane (75-28-5); USA – NJ – Right to Know Hazardous Substance List. Stoddard Solvent (8052-41-3); USA – NJ – Right to Know Hazardous Substance List. Benzene (71-43-2); USA – NJ – Right to Know Hazardous Substance List USA – PA – RTK List. Naphthalene (91-20-3); USA – NJ – Right to Know Hazardous Substance List USA – PA – RTK List. Cumene (98-82-8); USA – NJ – Right to Know Hazardous Substance List USA – PA – RTK List. Ethyl Benzene (100-41-4); USA – MA – Right to Know List USA – NJ – Right to Know Hazardous Substance List USA – PA – RTK List. Toluene (108-88-3); USA – MA – Right to Know List USA – NJ – Right to Know Hazardous Substance List USA – PA – RTK List. n-Heptane (142-82-5); USA – NJ – Right to Know Hazardous Substance List. |

16. OTHER INFORMATION

Indication of changes:

| Section | Changed item | Change |
| --- | --- | --- |
| 1 | SDS US Regulation Reference | Added |
| 2.1 | GHS-US Classification | Added |
| 2.2 | Hazard Pictograms (GHS-US) | Added |
| 2.2 | Precautionary statements (GHS-US) | Added |
| 2.2 | Hazard statements (GHS-US) | Added |
| 3 | Composition/Information on | Added |
| 4 | Symptoms/effects after ingestion | Modified |
| 4 | Symptoms/effects after skin contact | Modified |
| 4 | Symptoms/effects after eye contact | Modified |
| 4 | Other medical advice or treatment | Added |
| 4.1 | First-aid measures after skin contact | Modified |
| 4.1 | First-aid measures after inhalation | Modified |
| 4.1 | First-aid measures after eye contact | Modified |
| 4.1 | First-aid measures general | Added |
| 4.1 | First-aid measures after ingestion | Added |
| 7.2 | Storage conditions | Added |
| 8.2 | Compliance | Added |
| 8.2 | Remarks | Added |
| 8.2 | Hand Protection | Added |
| 8.2 | Environmental Exposure Controls | Added |
| 8.2 | Eye / Face Protection | Added |
| 8.2 | Skin and Body Protection | Added |
| 8.2 | Respiratory Protection | Added |
| 8.2 | Appropriate Engineering Controls | Added |
| 9 | Relative vapor density at 20°C | Added |
| 9 | Explosive properties | Added |
| 9 | Gas group | Added |
| 9 | Melting point | Added |
| 9 | Flash point | Added |
| 9 | Auto-ignition temperature | Added |
| 9 | Explosive limits (vol %) | Added |
| 9 | Appearance | Added |
| 9 | Specific gravity/density | Added |
| 10 | Incompatibilities | Added |
| 10 | Storage Requirements | Added |
| 10 | Possibilities of hazardous reactions | Added |
| 10 | Hazardous decomposition products | Added |
| 10 | Conditions to avoid | Added |
| 11 | Carcinogen status | Added |
| 12.1 | Ecology-general | Added |
| 14 | User precautions | Added |
| 14 | EmS Code *Column 15 in IMDG Book 2) | Added |
| 15 | Select the Appropriate Proposition 65 Notice | Added |
| 15 | Display TSCA summary in 15.1 | Added |
| 15 | Display SARA 313 summary in 15.1 | Added |
| 15 | Display California Proposition 65 summary in 15.3 | Added |

Full Text of H-Statements:

| H Code | H Phrase |
| --- | --- |
| H220 | Extremely flammable gas |
| H225 | Highly flammable liquid and vapour |
| H226 | Flammable liquid and vapour |
| H280 | Contains gas under pressure; may explode if heated |
| H304 | May be fatal if swallowed and enters airways |
| H315 | Causes skin irritation |
| H336 | May cause drowsiness or dizziness |
| H400 | Very toxic to aquatic life |
| H402 | Harmful to aquatic life |
| H410 | Very toxic to aquatic life with long lasting effects |
| H412 | Harmful to aquatic life with long lasting effects |

Disclaimer of Liability:	The information contained herein is based upon data provided to us by our suppliers, and reflects our best judgment.  However, no warranty of merchantability, fitness for any use, or any other warranty or guarantee is expressed or implied regarding the accuracy of such data, or the results to be obtained from us thereof.  Since the information contained herein may be applied under conditions beyond our control and with which we may be unfamiliar, we do not assume any responsibility for the results of such application.  This information is furnished upon the condition that the persons receiving it shall make their own determinations of the suitability of the material for any particular use.  Although certain hazards are described herein, we cannot guarantee these are the only hazards that exist.